State/District Skill Competitions

Test project: Sample

SKILL: BRICK LAYING

Skill-Brick Laying

Skill Explained

A bricklayer works on commercial and residential projects. The main responsibility of a bricklayer is to construct masonry components accurately and neatly according to the plan which often contains technical details. The bricklayer will interpret drawings, set out and measure, construct and finish to a high standard. Successful bricklayer requires endurance, concentration, an eye for details with continuing responsibility to work professionally in order to meet the requirements of the customer.

Eligibility Criteria- Competitors born on or after 01 Jan 1997 are only eligible to attend the Competition

Duration of Test project-8 hours

Preface

Section A-Test Project

Section B-Marking Scheme

Section C-Infrastructure List (Tool and equipment including raw material)

Section D- Instruction for Competitors

Section E- Health, Safety and Environment

Section- A

Drawing 2 – Details 2017

Drawing 1 – Elevation & Section

Section-B

The Assessment is done by awarding points by adopting two methods, Measurement and Judgments

- Measurement –One which is measurable

- Judgments-Based on Industry expectations

- Aspects are criteria’s which are judged for assessment

| Marking : Measurement |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Competitor Name - |  |  |  |  |  |  |
| Sub Criterion - Module – 2017 Trowel Designed Brick Work |  |  |  |  |  |  |
| Aspect ID | Aspect of Sub Criterion | Max Mark | Aspect of Sub Criterion – Description | Requirement of Size | Result or Actual Value | Mark Awarded |
| A1 | Dimension | 4 | Length of the bottom first layer at the top of bricks | 1830mm |  |  |
| A2 |  | 4 | Height of module | 1530mm |  |  |
| A3 |  | 4 | Height of bottom of zero to top of 2 | 606mm |  |  |
| A4 |  | 3 | Length of trowel triangle at top ( out to out ) | 355mm |  |  |
| A5 |  | 4 | Length of bottom point of 7 from left side of module | 920mm |  |  |
| B1 | Level | 3 | Top of number 7 | 0mm |  |  |
| B2 |  | 3 | Top of trowel traingle | 0mm |  |  |
| B3 |  | 3 | Top of module | 0mm |  |  |
| B4 |  | 2 | Top of number 2 | 0mm |  |  |
| B5 |  | 2 | Top of number 0 | 0mm |  |  |
| C1 | Alignment | 2 | Diagonal from layer 11 to bottom layer of number 7 | 0mm |  |  |
| C2 |  | 2 | Corner of number 1 to top of module right side | 0mm |  |  |
| C3 |  | 3 | bottom layer right side to top layer left side diagonally | 0mm |  |  |
| C4 |  | 3 | Alignment to 2017 diagonal | 0mm |  |  |
| CP1 | Plumb | 3 | Left side - front view | 0mm |  |  |
| CP2 |  | 2 | Left side - side view | 0mm |  |  |
| CP3 |  | 3 | Right side - front view | 0mm |  |  |
| CP4 |  | 2 | Right side - side view | 0mm |  |  |
| CP5 |  | 2 | Left side of number 2 | 0mm |  |  |
| CP6 |  | 2 | Right side of number 1 | 0mm |  |  |
| D1 | Details - 14 | 2 | Projection of number 7 at bottom | 10mm (+) |  |  |
| D3 |  | 2 | Projection of number 2 at top | 10mm (+) |  |  |
| D2 |  | 2 | Trowel projection at bottom | 10mm (-) |  |  |
| D4 |  | 2 | Projection at top of handle of trowel | 10mm (+) |  |  |
| D5 |  | 2 | Length of number 1 bottom from left side | 575mm |  |  |
| D6 |  | 2 | Height of number 7 | 375mm |  |  |
| D7 |  | 2 | Right angle at bottom each side | 90 degrees |  |  |
| Total Marks |  | 70 |  |  |  |  |

| Marking: Judgment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Competitor Name: |  |  |  |  |
| Sub Criterion: Module - 2017 Trowel Designed Brick Work |  |  |  |  |
| Aspect ID | Aspect Of Sub Criterion | Max Mark | Aspect of Sub Criterion - Description | Mark Awarded |
| J1 | Jointing | 10 | Uniformity of Joints for vertical and horizontal, regular joints size of 10mm, clean |  |
|  |  | 2 | Below Industry Standard |  |
|  |  | 4 | Industry Standard |  |
|  |  | 8 | Surpasses Industry Std. in some ways |  |
|  |  | 10 | Surpasses Industry Std. in most of the ways |  |
| J2 | Finishing | 10 | Overall Impression, Sharp Edge, Shape of Structure |  |
|  |  | 2 | Below Industry Standard |  |
|  |  | 4 | Industry Standard |  |
|  |  | 8 | Surpasses Industry Std. in some ways |  |
|  |  | 10 | Surpasses Industry Std. in most of the ways |  |
| J3 | Appearance | 5 | Cleaning Of Bricks and Overall Module |  |
| J4 |  | 5 | Cleaning Of Area Around |  |
| Total Marks |  | 30 |  |  |
| Remarks - |  |  |  |  |

Section-C

- Workshop Installation-Tools & Equipment positioned by Organizers

- Tool Kit-Tool & Equipment allowed to be brought by competitors for competitions

| Infrastructure List -Bricklaying |  |
| --- | --- |
| Workshop Installation | Quantity |
| Mortar Mixer | 1 |
| Water Tank | 1 |
| Water Bucket | 1 |
| Water Mug | 1 |
| Mortar Tub | 1 |
| Waste skip on wheels/Wheel barrow | 1 |
| Shovel for cleaning of area | 1 |
| Workshop Installation - Tools | Quantity |
| Spirit Level 300mm | 1 |
| Spirit Level 1m | 1 |
| Spirit Level 2m | 1 |
| Measuring Tape | 1 |
| Brick Cutting Machine/Brick Saw Table | 1 |
| Brick Trowels Big | 1 |
| Brick Trowel Small | 1 |
| Brick Hammer | 1 |
| Straight Edge 6 feet | 1 |
| Right angle big | 1 |
| Right Angle Small | 1 |
| Combination Square | 1 |
| Racker Smoother | 1 |
| Round Racker | 1 |
| Materials/ Consumables | Quantity |
| Plywood 1m * 1.5m (If table is possible) | 1 |
| Gauged Bond Paper | 1 |
| Scrubbing Brush for cleaning | 1 |
| Broom | 1 |
| Dustpan | 1 |
| Sponge | 2 |
| Line Dori | 1 bundle |
| Bricks 220*105*67 mm (hollow) required per module | 180 no.s |
| Bricks 220*105*67 mm yellow colour per module | 45 no.s |
| Mortar- Ready mix 40kg per bag | 4 bags (160kg) |
| Health & Safety Instruments | Quantity |
| Ear Plug | 1 pair |
| Knee pad | 1 pair |
| Nose mask | 1 |
| Goggle | 1 |
| Safety Shoes | 1 pair |
| Hand Gloves | 1 pair |
| Stationary | Quantity |
| Calculator | 1 |
| Scale 300mm | 2 |
| Protractor Small | 1 |
| Geometric Compass box | 1 |
| Marker Pen | 4 |

Section-D

- Time:   Drawing Reading – Either previous day or 1 hour before competition

Working Time – 8 Hours in two days 	Cleaning of Area – ½ hour after working time

1st Day – 5 hours

2nd Day – 3 hours and ½ hour after working time

- Drawing: Read the drawing carefully. Understand Dimensions, Projections and Cutting of bricks.

- Brick Sizes for all colors : 220 * 105 * 67 mm

Joint Size – 10mm bed joint (horizontal) and 5 mm racked (deep). Vertical joints to be in straight line

- Mark the color of bricks

- Projection: +/- 0 is our reference main wall. Consider + and – projection with reference to main wall. Refer Sectional and detail drawing.

- +/- 0 projections is for non-color bricks

- Kindly consider projection for 2017 as +10mm

- Trowel triangle projection (-10mm)

- Bottom of handle/ vertical part (+/- 0)

- Handle/Angular part - +10mm

- Health and Safety: All participants must follow health and safety measures, suggested to wear the issued protection equipment such as ear plugs, knee pads, goggles and safety shoes

- Planning: Before starting to the work kindly submit the planning stating activity and time for that

Section E

1. All accredited participants, and supporting volunteers will abide by rules and regulations with regards to Health, Safety, and Environment of the Competition venue.

2. All participants, technicians and supporting staff will wear the required protective Personnel clothing.

3. All participants will assume liability for  all risks of injury and damage to property, loss of property, which might be associated with or result from participation in the event.  The organizers will not be liable for any damage, however in case of Injury the competitor will immediately inform the immediate organizer for medical attention.